SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE CODE:  CSA0713                    COURSE NAME: COMPUTER NETWORKS

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

ANALYTICAL PROBLEM SOLVING

1. Suppose we want to transmit the message 11100011 and protect it from errors using the CRC polynomial X3+1.

i.) Use polynomial long division to determine the message that should be transmitted.

ii) Suppose the leftmost bit of the message is inverted due to noise on the transmission link. What is the result of the receiver's CRC calculation? How does the receiver know that an error has occurred?

2. a) A network with bandwidth of 10 Mbps can pass only an average of 12,000 frames per minute with each frame carrying an average of 10,000 bits. What is the throughput of this network?

b) A network has a bandwidth of 3000 Hz (300 to 3300 Hz) assigned for data communications. The signal- to-noise ratio is 3162. Find the channel capacity

3. A multispecialty hospital uses wireless patient-monitoring devices to continuously transmit patients' heart rate and blood pressure readings to a central monitoring station. Since the data is transmitted over a wireless channel, electromagnetic interference occasionally changes one bit during transmission. To ensure that doctors receive accurate patient information without waiting for retransmission, the hospital implements the Hamming (12,8) error-correcting code.

4. a) A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

b) A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

5. A university is conducting an online semester examination for 8,000 students simultaneously. At exactly 10:00 AM, the examination server begins transmitting encrypted question papers to all students. The university server is connected through a 10 Gbps high-speed network, while students access the examination portal using different internet connections such as fiber broadband (100 Mbps), home Wi-Fi (20 Mbps), and mobile networks (5–10 Mbps). During the first few minutes of the examination, many students with slower internet connections report incomplete downloads, repeated retransmissions, delayed acknowledgments, and temporary freezing of the examination portal. Network administrators observe that the server continues transmitting data at a high rate even though several student devices are unable to receive and process packets at the same speed. This results in receiver buffer overflow, unnecessary retransmissions, increased network traffic, and delays in delivering the question papers. Analyze the above scenario and identify the primary networking problem. Explain how the mismatch between the sender's transmission speed and the receiver's processing capability affects communication.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

ANALYTICAL PROBLEM SOLVING - SOLUTIONS

Answer 1: CRC Error Detection

Given: Message M = 11100011 (8 bits); Generator polynomial G(x) = x³ + 1, i.e. G = 1001 (4 bits, so r = 3 check bits).

(i) Determining the transmitted message

Step 1: Append r = 3 zero bits to the message before division:

11100011 + 000  =  11100011000

Step 2: Perform modulo-2 (XOR) division of 11100011000 by the generator 1001:

Carrying out the division bit by bit (XOR whenever the leading bit is 1) gives a final remainder of:

Remainder = 100

Therefore the transmitted codeword T = Message + Remainder:

T = 11100011  100  =  11100011100

(ii) Effect of the leftmost bit being inverted

The leftmost bit of the transmitted codeword (11100011100) is inverted:

Received codeword R = 01100011100

The receiver divides R by the same generator G = 1001 using modulo-2 division. This division yields:

Remainder at receiver = 010  (non-zero)

Since the remainder is not equal to 000, the receiver concludes that the received codeword is not exactly divisible by the generator polynomial. A non-zero remainder is the signal used by CRC to flag that transmission errors occurred, so the receiver detects the error and can request retransmission (or discard the frame).

Answer 2: Throughput and Channel Capacity

(a) Throughput calculation

Given: Bandwidth = 10 Mbps, Frames = 12,000 per minute, Frame size = 10,000 bits each.

Frames per second = 12,000 / 60 = 200 frames/sec

Throughput = Frames per second x Bits per frame

Throughput = 200 x 10,000 = 2,000,000 bps = 2 Mbps

Note: even though the link bandwidth is 10 Mbps, the effective (actual) throughput achieved is only 2 Mbps, i.e. only 20% of the available capacity is being utilised.

(b) Channel capacity calculation (Shannon’s theorem)

Given: Bandwidth B = 3000 Hz, Signal-to-Noise Ratio SNR = 3162.

Shannon’s Channel Capacity formula:

C = B x log₂(1 + SNR)

Substituting the values:

C = 3000 x log₂(1 + 3162) = 3000 x log₂(3163)

log₂(3163) ≈ 11.63

C ≈ 3000 x 11.63 ≈ 34,881 bps ≈ 34.88 kbps

Answer 3: Hamming (12,8) Error-Correcting Code

Why Hamming (12,8) suits the scenario: Patient-monitoring data (heart rate, blood pressure) is time-critical, so waiting for a retransmission (as in error-detection-only schemes like CRC/parity) is undesirable. Hamming codes are Forward Error Correction (FEC) codes: the receiver can locate and correct a single-bit error immediately, without asking the transmitter to resend the frame. Hence doctors keep receiving accurate readings continuously and in real time.

Structure of Hamming (12,8):

•  8 bits are the actual data (patient reading) and 4 additional parity (check) bits are added, giving a 12-bit codeword.

•  The 4 parity bits are placed at positions that are powers of 2: positions 1, 2, 4 and 8. The 8 data bits occupy the remaining positions: 3, 5, 6, 7, 9, 10, 11 and 12.

•  Each parity bit is computed as the XOR (even parity) of a specific subset of positions, chosen so that every bit position’s binary index tells exactly which parity bits cover it:

•  p1 (position 1) checks positions 1, 3, 5, 7, 9, 11

•  p2 (position 2) checks positions 2, 3, 6, 7, 10, 11

•  p4 (position 4) checks positions 4, 5, 6, 7, 12

•  p8 (position 8) checks positions 8, 9, 10, 11, 12

Illustrative example: encoding 8 data bits d1..d8 = 1 0 1 1 0 0 1 0

| Position | Bit | Covering parity bits |
| --- | --- | --- |
| 1 (p1) | 1 | — (parity bit) |
| 2 (p2) | 0 | — (parity bit) |
| 3 (d1) | 1 | p1, p2 |
| 4 (p4) | 0 | — (parity bit) |
| 5 (d2) | 0 | p1, p4 |
| 6 (d3) | 1 | p2, p4 |
| 7 (d4) | 1 | p1, p2, p4 |
| 8 (p8) | 1 | — (parity bit) |

(Positions 9-12 = d5 d6 d7 d8 = 0 0 1 0, covered similarly by p8 together with p1/p2/p4 as applicable.)

Computing each parity bit as the XOR of the data bits it covers gives: p1 = 1, p2 = 0, p4 = 0, p8 = 1.

Resulting 12-bit transmitted codeword (positions 1-12):

1 0 1 0 0 1 1 1 0 0 1 0

Error detection and correction at the receiver:

Suppose electromagnetic interference flips the bit at position 7 during transmission, so the receiver gets: 1 0 1 0 0 1 0 1 0 0 1 0.

The receiver recomputes each parity check (p1, p2, p4, p8) over the received bits. Every check that fails contributes its position value to a "syndrome":

Syndrome = p1(fail=1) + p2(fail=1) + p4(fail=0) + p8(fail=1) = 1 + 2 + 0 + 4 = 7

The syndrome value 7 directly points to position 7 as the location of the error. The receiver simply flips bit 7 back, recovering the correct 12-bit codeword and the correct 8 data bits — without needing any retransmission.

Answer 4: IPv4 Subnetting and IPv6 Addressing

(a) IPv4 subnetting: 172.16.10.0/24 with /27 mask

Subnet mask /27 = 255.255.255.224. Bits borrowed from the host portion = 27 - 24 = 3.

Number of subnets = 2³ = 8 subnets

Host bits remaining = 32 - 27 = 5, so hosts per subnet = 2⁵ - 2 = 30 usable hosts (subtracting the network and broadcast addresses).

| Subnet # | Network Address | Usable Host Range | Broadcast Address |
| --- | --- | --- | --- |
| 1 | 172.16.10.0/27 | 172.16.10.1 - .30 | 172.16.10.31 |
| 2 | 172.16.10.32/27 | 172.16.10.33 - .62 | 172.16.10.63 |
| 3 * | 172.16.10.64/27 | 172.16.10.65 - .94 | 172.16.10.95 |
| 4 | 172.16.10.96/27 | 172.16.10.97 - .126 | 172.16.10.127 |
| 5 | 172.16.10.128/27 | 172.16.10.129 - .158 | 172.16.10.159 |
| 6 | 172.16.10.160/27 | 172.16.10.161 - .190 | 172.16.10.191 |
| 7 | 172.16.10.192/27 | 172.16.10.193 - .222 | 172.16.10.223 |
| 8 | 172.16.10.224/27 | 172.16.10.225 - .254 | 172.16.10.255 |

* 172.16.10.78 lies between .64 and .95, so it belongs to Subnet 3: 172.16.10.64/27, with network address 172.16.10.64 and broadcast address 172.16.10.95.

172.16.10.95 is the broadcast address of that same subnet (172.16.10.64/27), so it falls within the same subnet’s address range — however, as a broadcast address it is not itself a usable host address.

(b) IPv6 addressing: 2001:0db8:0000:0000:0000:ff00:0042:8329

Compression rules: (1) leading zeros within each 16-bit group may be omitted, and (2) one contiguous run of all-zero groups may be replaced with "::" (only once per address).

Compressed form:  2001:db8::ff00:42:8329

Expanding the compressed address back to full notation (restoring the zero groups and leading zeros):

Full form:  2001:0db8:0000:0000:0000:ff00:0042:8329

Network prefix with /64: the first 64 bits (first 4 groups) form the network prefix:

Network prefix = 2001:0db8:0000:0000::/64  (i.e. 2001:db8::/64)

Number of possible host addresses in this /64 network = remaining 128 - 64 = 64 host bits:

Total host addresses = 2⁶⁴ = 18,446,744,073,709,551,616 (≈ 1.8 x 10¹⁹) addresses

Answer 5: Analysis of the Examination Server Scenario

Primary networking problem: This is fundamentally a Flow Control problem, compounded by network congestion. The exam server (10 Gbps) keeps sending encrypted question papers at a high, constant rate to 8,000 students simultaneously, without adapting to the fact that many receivers (home Wi-Fi at 20 Mbps, mobile networks at 5-10 Mbps) can accept and process data far more slowly than the sender can transmit it.

How the sender-receiver speed mismatch causes the observed symptoms:

•  Receiver buffer overflow: Each student’s device/router has a limited receive buffer. When the server sends data faster than a slow connection can drain it, the buffer fills up and additional incoming packets are dropped.

•  Unnecessary retransmissions: Dropped packets are detected as missing/timed-out at the receiver, triggering retransmission requests, which adds even more traffic onto already-congested links.

•  Delayed acknowledgements and freezing: Because the receiver is busy discarding/recovering from overflowed data, ACKs are delayed, which stalls the sender’s subsequent transmissions and makes the portal appear frozen to the student.

•  Increased network traffic: With 8,000 simultaneous sessions plus the extra retransmissions, the shared network experiences congestion, further increasing delay and packet loss for everyone — a feedback loop that worsens the problem over time.

Distinguishing Flow Control vs Congestion Control:

•  Flow control is an end-to-end (sender-receiver) issue: it ensures a fast sender does not overwhelm a slower receiver, typically using mechanisms like sliding-window protocols or TCP’s receiver-advertised window, so the sender only transmits as much data as the receiver can currently buffer.

•  Congestion control is a network-wide issue: it prevents the shared network itself (routers/links) from being overloaded by traffic from many sources, using mechanisms such as TCP’s slow start, congestion avoidance, and Additive-Increase-Multiplicative-Decrease (AIMD).

•  In this scenario both are involved: the immediate symptom (buffer overflow at each student’s device) is a flow-control failure, while the resulting spike in retransmissions across 8,000 concurrent sessions produces network-wide congestion.

Recommended fixes: implement proper flow control (receiver-window based transmission, e.g. TCP’s built-in flow control, so the server throttles its rate to match each student’s reported window size); apply congestion-control/avoidance algorithms and traffic shaping at the server; stagger or batch the release of question papers instead of a single synchronized 10:00 AM broadcast; and use caching/CDN or multicast distribution so the server is not individually pushing full-rate data to all 8,000 students at once.